Power Supply Test Report

Company: VMMP Group LTD.

Customer: CAP- CO LTD.

Model Name: GLB-3000W	List Date:	2018-9-14	16:47:26

Serial No :	000001	Test Date: 2018-9-14	16:47:26

================================ Test Condition ===============================

Retry Time: 2.00Sec		Ready Time: 1.00Sec		Steady Time: 0.60Sec Short Model: After Shorted	Input Select: AC to DC	Power Down Model: ATX Power

| VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Down Voltage | 8.00 | 8.00 | 3.00 | 3.00 | 8.00 | 8.00 | 8.00 | 2.00 | 3.00 8.00 |  |
| Reset Voltage | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 1.00 |  |

================================ Outcome Report ==============================

[01][13]AC 200V

[Pass]

[02][20]3Sec max

First Reference Voltage: 0.50V	Second Reference Voltage: 4.75V VxSelect: +5V-B	Turn on way: PS ON

VxName	+12V-I	+12V-A    +5V-B	+12V-C    +12V-G   +12V-H   -12V	+3.3V	+5VSB	+12V-J

Load Current(A)	26.00	27.00	11.00	26.00	27.00	26.00	0.500	11.00	2.500	26.00

Upper(ms)	25.00

Lower(ms)	0.20

Output(ms)	5.80

[Pass]

[03][10]100% LOAD

Wait Times: 0.5S	Wait Select: Wait Pass

| VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load Current(A) | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 26.00 |  |
| Upper(V) | 12.600 | 12.600 | 5.250 | 12.600 | 12.600 | 12.600 | 12.600 | 3.470 | 5.250 12.600 |  |
| Lower(V) | 11.400 | 11.400 | 4.750 | 11.400 | 11.400 | 11.400 | 11.400 | 3.140 | 4.750 11.400 |  |
| Output(V) | 12.06 | 12.01 | 4.97 | 11.90 | 12.08 | 12.05 | 12.05 | 3.21 | 4.96 12.07 |  |
| Output(W) | 313.56 | 324.27 | 54.67 | 309.40 | 326.16 | 313.30 | 6.03 | 35.31 | 12.40 313.82 |  |

Total Power(W)	2008.915

[Pass]

[04][13]AC 230V

[Pass]

[05][11]+12V SHORT

Short Times: 0.50S	Down Model: AND	VxSelect: +12V-A Turn Off Way: PS OFF

| VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load Current(A) | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 26.00 |  |
| Upper(V) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 2.00 | 5.25 3.00 |  |
| Lower(V) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 4.50 0.00 |  |
| Output(V) [Pass] | 0.04 | 0.03 | 0.35 | 0.03 | 0.03 | 0.04 | 0.75 | 0.23 | 5.06 0.04 |  |
| [06][16]PS ON VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| Load Current(A) [Pass] | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 26.00 |  |

[07][11]-12V SHORT

Short Times: 0.30S	Down Model: AND	VxSelect: -12V Turn Off Way: PS OFF

| VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load Current(A) | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 26.00 |  |
| Upper(V) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 2.00 | 5.25 3.00 |  |
| Lower(V) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 4.50 0.00 |  |
| Output(V) [Pass] | 0.75 | 0.71 | 0.66 | 0.73 | 0.74 | 0.74 | 0.03 | 0.87 | 5.06 0.75 |  |
| [08][16]PS ON VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| Load Current(A) [Pass] | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 26.00 |  |

[09][11]+5V SHORT

Short Times: 0.30S	Down Model: AND	VxSelect: +5V-B Turn Off Way: PS OFF

| VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB |  | +12V-J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load Current(A) | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 | 26.00 |  |
| Upper(V) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 2.00 | 5.25 | 3.00 |  |
| Lower(V) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 4.50 | 0.00 |  |
| Output(V) [Pass] | 0.73 | 0.73 | 0.00 | 0.75 | 0.76 | 0.75 | 0.80 | 0.88 | 5.06 | 0.73 |  |
| [10][16]PS ON VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB |  | +12V-J |
| Load Current(A) [Pass] | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 | 26.00 |  |
| [11][11]+3.3V SHORT Short Times: 0.30S Turn Off Way: PS OFF VxName +12V-I |  | Down Model: AND +12V-A +5V-B +12V-C +12V-G |  |  |  | +12V-H | VxSelect: +3.3V -12V +3.3V +5VSB |  |  | +12V-J |  |
| Load Current(A) 26.00 |  | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 | 26.00 |  |

| Upper(V) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 2.00 | 5.25 | 3.00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lower(V) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 4.50 | 0.00 |  |
| Output(V) [Pass] | 0.75 | 0.75 | 0.65 | 0.77 | 0.77 | 0.74 | 0.82 | 0.00 | 5.06 | 0.76 |  |
| [12][16]PS ON VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB |  | +12V-J |
| Load Current(A) [Pass] | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 | 26.00 |  |
| [13][17]PS OFF [Pass] |  |  |  |  |  |  |  |  |  |  |  |

[14][03]Power Good Time Test

Time Reference Voltage: 4.75V	Turn On Way: PS ON

VxName	+12V-I	+12V-A    +5V-B	+12V-C    +12V-G   +12V-H   -12V	+3.3V	+5VSB	+12V-J

Load Current(A)	26.00	27.00	11.00	26.00	27.00	26.00	0.500	11.00	2.500	26.00

Upper(ms)	500.0

Lower(ms)	100.0

Output(ms)	256.1

[Pass]

[15][08]OPP TEST

Step Times: 0.15S	Down Model: AND	Turn Off Way: PS OFF

| VxName | +12V-I +12V-A +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load Current(A) | 26.00 27.00 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 26.00 |  |
| Step Current(A) | 1.00 1.00 1.00 | 1.00 | 1.00 | 1.00 | 0.010 | 1.00 | 0.00 1.00 |  |
| End Current(A) Upper Lower Output [Pass] | 60.00 60.00 60.00 3200.0W 5.50V 2400.0W 0.50V 2871.5W | 60.00 5.06V | 60.00 | 60.00 | 4.000 | 60.00 | 8.000 60.00 |  |
| [16][16]PS ON VxName | +12V-I +12V-A +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| Load Current(A) [Pass] | 26.00 27.00 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 26.00 |  |
| [17][13]AC 264V [Pass] |  |  |  |  |  |  |  |  |
| [18][21]PF TEST Wait Times: 2.50S VxName | +12V-I +12V-A +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| Load Current(A) | 26.21 26.21 11.48 | 26.21 | 26.21 | 26.21 | 0.480 | 11.48 | 2.380 26.21 |  |
| Meter Value | Freq Ip-p Watt | Vrms | Irms | Eff | Vp-p | P.F. |  |  |
| Upper | 63.00 15.00 2600.00 | 400.00 | 10.00 | 100.00 | 500.00 | 1.00 |  |  |
| Lower | 0.00 0.00 0.00 | 0.00 | 0.00 | 87.00 | 0.00 | 0.950 |  |  |
| Output | 50.00 11.96 2217.90 | 264.40 | 8.40 | 91.48 | 378.00 | 1.000 |  |  |

[Pass]

[19][13]AC 230V

[Pass]

[20][10]100%                       LOAD 敲 打 /       看 波 形 、Wait Times: 0.5S          Wait Select: Wait

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   26.00   27.00   11.00   26.00   27.00   26.00    0.500    11.00    2.500    26.00 Upper(V)      12.600   12.600   5.250    12.600   12.600   12.600   12.600   3.470    5.250    12.600 Lower(V)     11.400   11.400   4.750    11.400   11.400   11.400   11.400   3.140    4.750    11.400 Output(V)     12.06    12.01    4.98     11.90    12.07    12.05    12.10    3.22     4.96     12.06 Output(W)        313.56   324.27   54.78    309.40   325.89   313.30   6.05     35.42    12.40    313.56 Total Power(W)   2008.630

[Pass]

[21]Ripple&Noise Test

| VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load Current(A) | 26.00 | 27.00 | 11.00 | 26.00 | 27.00 | 26.00 | 0.500 | 11.00 | 2.500 26.00 |  |
| V-Upper(V) | 12.60 | 12.60 | 5.25 | 12.60 | 12.60 | 12.60 | 12.60 | 3.47 | 5.25 12.60 |  |
| V-Lower(V) | 11.40 | 11.40 | 4.75 | 11.40 | 11.40 | 11.40 | 11.40 | 3.14 | 4.75 11.40 |  |
| Output(V) | 12.06 | 12.01 | 4.98 | 11.90 | 12.07 | 12.05 | 12.10 | 3.22 | 4.96 12.06 |  |
| R & N Upper(mV) | 120 | 120 | 50 | 120 | 120 | 120 | 120 | 50 | 50 120 |  |
| R & N Lower(mV) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 0 |  |
| Output(mV) | 78 | 75 | 36 | 74 | 81 | 84 | 83 | 34 | 32 78 |  |
| [Pass] |  |  |  |  |  |  |  |  |  |  |

[22][01]+12V MAX LOAD

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   27.50   27.50   0.10   27.50    27.50    27.50    0.100    0.10    0.100    27.50 Upper(V)       12.60    12.60    5.25     12.60    12.60    12.60    12.60    3.47     5.25     12.60 Lower(V)      11.40    11.40    4.75     11.40    11.40    11.40    11.40    3.14     4.75     11.40 Output(V)     12.05    12.01    5.02     11.88    12.08    12.05    12.11    3.28     5.05     12.06 Output(W)        331.38   330.28   0.50     326.70   332.20   331.38   1.21     0.33     0.51     331.65 Total Power(W)   1986.12

[Pass]

[23][01]+5V MAX LOAD

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   26.00   27.00   20.00   26.00   27.00    26.00    0.100    0.00    0.100    26.00 Upper(V)       12.60    12.60    5.25     12.60    12.60    12.60    12.60    3.47     5.25     12.60 Lower(V)      11.40    11.40    4.75     11.40    11.40    11.40    11.40    3.14     4.75     11.40 Output(V)     12.06    12.01    4.96     11.90    12.08    12.06    12.12    3.26     5.04     12.07 Output(W)        313.56   324.27   99.20    309.40   326.16   313.56   1.21     0.00     0.50     313.82 Total Power(W)   2001.69

[Pass]

[24][01]+3.3V MAX LOAD

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   26.00   27.00   6.00   26.00   27.00    26.00    0.100    20.00    0.100    26.00 Upper(V)       12.60    12.60    5.25     12.60    12.60    12.60    12.60    3.47     5.25     12.60 Lower(V)      11.40    11.40    4.75     11.40    11.40    11.40    11.40    3.14     4.75     11.40 Output(V)     12.06    12.01    4.98     11.89    12.07    12.05    12.12    3.17     5.04     12.06 Output(W)        313.56   324.27   29.88    309.14   325.89   313.30   1.21     63.40    0.50     313.56 Total Power(W)   1994.72

[Pass]

[25][01]LOW LOAD

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   0.00    0.15    0.20    0.00    0.00     0.00     0.000    0.10     0.000    0.00 Upper(V)       12.60    12.60    5.25     12.60    12.60    12.60    12.60    3.47     5.25     12.60 Lower(V)      11.40    11.40    4.75     11.40    11.40    11.40    11.40    3.14     4.75     11.40 Output(V)     12.24    12.24    5.11     12.23    12.24    12.23    12.01    3.37     5.15     12.23 Output(W)         0.00     1.84     1.02     0.00     0.00     0.00     0.00     0.34     0.00     0.00 Total Power(W)   3.20

[Pass]

[26]Ripple&Noise Test

| VxName | +12V-I | +12V-A | +5V-B | +12V-C | +12V-G | +12V-H | -12V | +3.3V | +5VSB | +12V-J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load Current(A) | 0.00 | 0.15 | 0.20 | 0.00 | 0.00 | 0.00 | 0.000 | 0.10 | 0.000 0.00 |  |
| V-Upper(V) | 12.60 | 12.60 | 5.25 | 12.60 | 12.60 | 12.60 | 12.60 | 3.47 | 5.25 12.60 |  |
| V-Lower(V) | 11.40 | 11.40 | 4.75 | 11.40 | 11.40 | 11.40 | 11.40 | 3.14 | 4.75 11.40 |  |
| Output(V) | 12.24 | 12.24 | 5.11 | 12.23 | 12.24 | 12.23 | 12.01 | 3.37 | 5.15 12.23 |  |
| R & N Upper(mV) | 120 | 120 | 50 | 120 | 120 | 120 | 120 | 50 | 50 120 |  |
| R & N Lower(mV) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 0 |  |
| Output(mV) | 20 | 19 | 10 | 14 | 18 | 14 | 21 | 12 | 13 18 |  |
| [Pass] |  |  |  |  |  |  |  |  |  |  |

[27][01]50% LOAD

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   13.00   14.00   5.50   13.00    13.00    13.00    0.250    5.50    1.250    13.00 Upper(V)       12.60    12.60    5.25     12.60    12.60    12.60    12.60    3.47     5.25     12.60 Lower(V)      11.40    11.40    4.75     11.40    11.40    11.40    11.40    3.14     4.75     11.40 Output(V)     12.15    12.12    5.04     12.07    12.17    12.15    12.05    3.29     5.06     12.16 Output(W)        157.95   169.68   27.72    156.91   158.21   157.95   3.01     18.10    6.33     158.08 Total Power(W)   1013.93

[Pass]

[28][21]100%EFF

Wait Times: 2.50S

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   26.21    26.21    11.48    26.21    26.21    26.21    0.480    11.48    2.380   26.21

| Meter Value | Freq | Ip-p | Watt Vrms | Irms | Eff | Vp-p | P.F. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper | 63.00 | 25.00 | 2600.00 400.00 | 20.00 | 100.00 | 500.00 | 1.00 |  |
| Lower | 47.00 | 0.00 | 1300.00 90.00 | 0.00 | 87.00 | 90.00 | 0.950 |  |
| Output | 50.00 | 13.88 | 2227.60 230.40 | 9.68 | 91.30 | 330.10 | 1.000 |  |
| [Pass] |  |  |  |  |  |  |  |  |

[29][21]50%EFF		 Wait Times: 2.50S

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   13.11    13.11    5.74     13.11    13.11    13.11    0.240    5.74     1.190   13.11

| Meter Value | Freq | Ip-p | Watt Vrms | Irms | Eff | Vp-p | P.F. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper | 63.00 | 15.00 | 1200.00 400.00 | 10.00 | 100.00 | 500.00 | 1.000 |  |
| Lower | 47.00 | 0.00 | 800.00 90.00 | 0.00 | 90.00 | 90.00 | 0.500 |  |
| Output | 50.00 | 6.91 | 1095.80 231.00 | 4.74 | 92.94 | 330.20 | 1.000 |  |
| [Pass] |  |  |  |  |  |  |  |  |

[30][21]20%EFF		 Wait Times: 2.00S

| Freq | Ip-p | Watt | Vrms | Irms | Eff | Vp-p | P.F. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63.00 | 15.00 | 800.00 | 400.00 | 8.00 | 100.00 | 500.00 | 1.000 |  |
| 47.00 | 0.00 | 150.00 | 90.00 | 0.00 | 87.00 | 90.00 | 0.500 |  |
| 50.00 | 2.80 | 445.00 | 231.30 | 1.93 | 91.91 | 330.20 | 1.000 |  |

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   5.00     6.45     2.30     5.00     5.00     5.00     0.100    2.30     0.480    5.00 Meter Value

Upper

Lower

Output	 [Pass]

[31][06]Power Fail Time Test

Time Reference Voltage: 4.75V          Turn Off Way: PS OFF

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   26.00   27.00   11.00   26.00   27.00   26.00   0.500   11.00    2.500    26.00 Upper(ms)        50.0

Lower(ms) 1.0 Output(ms)    4.6 [Pass]

[32][16]PS ON

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   26.00   27.00   11.00    26.00    27.00    26.00    0.500    11.00    2.500    26.00 [Pass]

[33][10]100%	 Wait Times: 0.5S          Wait Select: Wait

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   26.00   27.00   11.00   26.00   27.00   26.00    0.500    11.00    2.500    26.00 Upper(V)      12.600   12.600   5.250    12.600   12.600   12.600   12.600   3.470    5.250    12.600 Lower(V)     11.400   11.400   4.750    11.400   11.400   11.400   11.400   3.140    4.750    11.400 Output(V)     12.06    12.01    4.98     11.90    12.07    12.05    12.09    3.22     4.96     12.06 Output(W)        313.56   324.27   54.78    309.40   325.89   313.30   6.05     35.42    12.40    313.56 Total Power(W)   2008.625

[Pass]

[34][05]Hold-On Time Test

Time Reference Voltage: 4.75V          Turn Off Way: AC OFF

VxName            +12V-I   +12V-A   +5V-B    +12V-C   +12V-G   +12V-H   -12V     +3.3V    +5VSB    +12V-J Load Current(A)   19.75   19.75   8.25   19.75   19.75   19.75    0.375    8.25    1.880    19.75 Upper(ms)        100.0

Lower(ms) 17.0 Output(ms)    18.6 [Pass]

[35][15]AC OFF                                     	 	 [Pass]

Equipment Support :Sun Moon Technlogy Corp.

Version:2.01-----------------------------------------------

End Of

Data	2

018-9-14 16:48:29

Total time:  63 (Sec)